Soil sample analysis protocol

Objectives

- Measure bulk density and gravimetric soil moisture

- Prepare and send samples to Waypoint Analytical

Materials

- 90 quart-sized plastic bags

- 180 paper bowls

- Graduated cylinder

- Mass balance

- 90-180 centrifuge tubes

(Done) Part 1: Lab Prep

- Prepare soil sample submittal form for Waypoint Analytical.

- Label soil sample bags, including date, plot number, quadrat number, and purpose.

(Done) Part 2: Measure bulk density and soil moisture (Al-shammary et al., 2018; Poeplau et al., 2017)

- Remove bulk density samples from plastic bags.

- Record total mass and place in labelled bowls (Msoil + Mwater + Mcoarse rocks/roots).

- Place bulk density samples in the oven at 105°C, dry for 24 hours, and record mass (Msoil + Mcoarse rocks/roots).

- Estimate mass of water as difference in mass between steps 1 and 2.

- Use sieve and hand removal to collect coarse roots and rocks (>2 mm).

- Use graduated cylinder to estimate volume (Vcoarse roots/rocks).

- Re-dry coarse roots and rocks (>2 mm) at 60°C for 48 hours.

- Re-weigh to estimate Mcoarse rocks/roots and subtract this from mass of soil plus coarse rocks and roots

- Estimate bulk density (g cm-3) as the mass of fine dry soil (Msoil) divided by the corrected volume of the cylinder (Vcore – Vcoarse roots/rocks).

- Estimate gravimetric soil moisture (g water g soil-1) as the mass of water (Mwater) divided by the mass of dry soil (Msoil).

(Done) Part 3: Send soil samples to Waypoint

- Homogenize soil in each gallon bag.

- Scoop 2 cups of soil into a quart-sized bag and weigh.

- Label each bag and place in box for transportation.

- Drop samples with complete submittal form off at 2902 Farber Dr, Champaign, IL, 61822 (Phone: 217-359-7680, Fax: 217-359-7605, Email: supportil@waypointanalytical.com)

Part 4: Fine roots

- Pass all remaining soil through 2 mm sieve and collect fine roots and fragments.

- Remove recognizable material < 2 mm by hand.

(Done) Part 5: Aggregate separation protocol (Maietta et al., 2019; Six et al., 2000)

Objective: Separate soil into very large macroaggregates (≥4.75 mm), large macroaggregates (≥2 mm), small macroaggregates (≥250 μm to 2000 μm), microaggregates (≥53 to <250 μm), and free mineral-associated organic matter (MAOM) (<53 μm)

Materials

- Many large and small aluminum tins

- 8-mm, 4.75-mm, 2000-µm, 250- µm and 53-µm sieves

- Four 5-gallon buckets

(Done) Step 5a: Wet-sieving and drying steps

- Pass field moist soil through an 8-mm sieve and then air-dry for 7 days at room temperature.

- Pre-weigh and label all aluminum tins.

- Weigh 100 g of each soil sample into aluminum tin and label with plot number.

- Place 100 g of air-dried soils on top of a 4.75-mm sieve and then submerge in distilled water for 5 min (slaking)

- After 5-min of slaking, separate aggregates by moving the sieve up and down through the distilled water column (~60 repetitions over 2 min).

- Transfer soils on top of the 4.75-mm sieve to a pre-weighed aluminum tin.

- Skim floating POM (fPOM) from the water surface and store in separate pre-weighed tin.

- Sequentially separate material that passed through the 4.75 mm sieve with 2000-, 250-, and 53-µm sieves

- Air-dry each sample until all water evaporated.

- Oven-dry samples at 60°C for 48 hours and then weigh samples to nearest 0.01 g.

(Done) Step 5b: Post-processing steps

- Pass each sample through 2 mm sieve to remove coarse rocks, roots, or litter fragments.

- Subtract mass from measured dry weight of each fraction.

- Subtract total mass of fragments from starting weight ~100 g.

Part 6: Total C and total organic C in bulk soil

- Weigh ~25 g of air-dry soil (<8 mm) into soil tins for each sample.

- Gently grind each sample to pass 2-mm sieve and remove all rocks/fragments/roots (>2 mm)

- Add 1-2 g of sample to 2-mL centrifuge tubes.

- Use shaker to grind samples to a powder.

- Oven-dry ground samples at 60°C for 48 hours.

- Weigh and wrap samples in foil for CHN analysis at IGB.

Part 7: Size separation between POM and MAOM. Adapted from Marriot and Wander (2006).

Materials

- 53-µm mesh fabric

- 5% Na hexametaphosphate solution

- Reciprocal shaker

- 30-mL plastic bottles

- 150-mL centrifuge tubes

Steps

- Weigh 20 g air-dry soil (<2 mm) into 30-mL plastic bottles

- Cover the mouth of the bottle with 53 um mesh fabric and then seal with caps with 2-cm holes drilled in their tops

- Place 30-mL bottle in 250-mL centrifuge bottle with 150 mL of 5% Na hexametaphosphate solution

- Shake samples from 1 h on a reciprocal shaker at ~180 oscillations/min.

- Replace Na hexametaphosphate and suspended fine particles with 150 mL tap water and re-shake for 20 min (repeat 2 times).

- Discard 150 mL of tap water and suspended fine particles.

- Transfer the POM to a larger piece of 53 um mesh fabric and rinse with DI water.

- Dry the POM at 60°C for 48 hours and then grind to a powder.

- Analyze for total C/N with elemental analyzer.

- Rinse the coarse fraction (CF) with 50 mL DI water